Instruction Manual for download retrieval

USB Host Adapter (download device)

How to use the adapter

- Make sure the Carrier reefer machinery is switched OFF 100%.

- Slide the download device into the controller (arrow up and logo towards condenser fan)

- Insert the USB stick into the USB connector of the download device.

- Switch ON the Carrier reefer machinery.

- Scroll with the arrows until “RETR DATA” is shown on the display.

- Press “enter”.

- Scroll with the arrows until the requested period is shown on the display (below are the options).

- Load all

- Load 90 days

- Load 60 days

- Load 30 days

- Load trip

- Scroll to 90 days and press “enter”.

- While retrieving the data, the Display (left window) shows “RETR ”; the display (right window) shows “percentage counter”.

- Display, when finished (100%), shows “RETR DONE”.

- Switch OFF the Carrier reefer machinery.

- Remove download device from the controller.

- Make sure container is switched ON again.

- Once unit is switched back ON, proceed with manual defrost.

- Remove the USB stick from download device.

- Take the USB stick to your pc and locate the *.DCX file(s).

- Email the file(s) to email address coldtreatment@medlog.be